Yorkshire Water Drought Permit Application

Exceptional Shortage of Rainfall Evidence

period

September 2022

| V1 | August 2022 | Submitted with NW pre application |
| --- | --- | --- |
| V2 | August 2022 | Resubmitted with NW pre application |
| 13-09-2022 | September 2022 | Updated with August data, and updated following NW Pre app comments. Submitted with NW application |
| V4 | September 2022 | For Ouse Application- Analyses redone for regional using EA YW region and incorporating comments from Ouse pre-app |
| V5 | September 2022 | Prepared for South pre application but not used-removed some of the analyses for NW and Ouse, and additional analyses for reservoirs in south and south west area. |
| V6 | October 2022 | As V 5 updated with September data. To be submitted for south and SW pre apps |
| YW Exceptional shortage of rain evidence_v7-09-2022_wharfe app | October 2022 | For Wharfe application, updated from v6 for all analyses. |
| YW Exceptional shortage of rain evidence_v8-2022 | October 2022 | Update correcting river catchment analyses for September and incorporating EA comments |

Contents

1.	Introduction	3

2.	2022 Weather Summary	3

3.	Rainfall data	4

4.	Rainfall and climate data analyses	5

5.	Rainfall as percentage of Long Term Average (LTA)	5

6.	Ranking of rainfall	7

7.	Soil Moisture Deficit (SMD)	8

8.	Standardised Precipitation Index (SPI)	9

9.	River Flows	11

10.	Reservoir stocks	15

11.	Weather Forecast	18

12.	Conclusions	18

13.	References	19

Appendices	20

Appendix 1: Rainfall in comparison to LTA	20

Appendix 2a: Ranking of Rainfall Results	34

Appendix 2b: Driest period on record	42

Appendix 2c: Last Year drier than 2021/22 for durations 1-12 months	49

Appendix 3: SPI graphs and maps from CEH Water Resources portal	58

Appendix 4: River flow graphs from CEH Water Resources portal	68

## Introduction

Yorkshire Water are applying for a number of drought permits and orders to enable us to deal with the serious risk of a deficiency of supplies that will exist if the dry weather continues throughout the autumn and winter of 2022/2023.  The spring and summer of 2022 has been one of very low rainfall and high temperatures.  This document outlines our analyses of rainfall data to date (and projections for the coming months) and demonstrates that the serious risk of deficiency of supplies is due to an exceptional shortage of rainfall.

The legal criteria that must be met in granting a drought order or permit include:

'If the Secretary of State / Agency is satisfied that, by reason of an exceptional shortage of rain, a serious deficiency of supplies of water in any area exists or is threatened…..'

There is no set definition or methodology to determine that an “exceptional shortage of rainfall” has occurred, because each drought and situation is unique.

In this document we demonstrate an exceptional shortage of rainfall by analysis of monthly rainfall and other metrics.

We have included:

Plots of 2022 rainfall in relation to long term average

Calculation of Cunnane Plotting position for long time series

MORECS Soil Moisture Deficit analysis

Standard Precipitation Indices

This document summarises the analyses carried out for the region and for catchments, and is a supplementary document to accompany the supporting information for our drought permit applications.  Each application refers to the information in this document, and highlights some of the relevant analyses.  Each application also shows relevant river flows and reservoir stocks relating to that application.

## 2022 Weather Summary

2022 started dry, with below average rainfall from November 2021 until January 2022.  February 2022 was extremely wet, with more than double the long term average (LTA), but all months had been below average since March until September 2022, which had close to average rainfall.  There have been periods of extremely hot weather, with heatwaves in July and early August, and the first ever red weather warnings for extreme heat in July.  It has been the driest July in England since 1935 (Met Office, 2022), with virtually no rainfall in the south east of the country.  In Yorkshire, there has been far less rainfall than usual, and this, combined with the high temperatures has meant that river flows and reservoir inflows have been far lower than usual.

The heatwave conditions resulted in exceptionally high demand for water.  July rainfall was lower than average in Yorkshire, but much of the rainfall for the month fell in the last 10 days of the month. The combination of hot dry periods with occasional rainfall has meant that much of the rainfall was soaked up by the dry ground. September has close to average rainfall, and this has improved the position, but there is still a considerable rainfall deficit.

## Rainfall data

We have used the HADUK river catchment rainfall data provided to us monthly by the Environment Agency.  In addition, we have used monthly catchment rainfall data for several of our reservoir group catchments throughout the region.  The catchments we have analysed are listed in Table 1 .  These data have been provided from the HADUK/EA by the EA NE regional hydrology team.  The use of the HADUK rainfall data allows analysis of long time series since it starts in 1891.

Figure 1 shows the Yorkshire River catchments and all of our reservoir catchments on a map, with the reservoir catchments we have analysed rainfall data for in this report labelled.  It is clear how small the reservoir catchments are in proportion to even the smaller river catchments.

| Catchment rainfall | Code/description | Source |
| --- | --- | --- |
| Wharfe | TPD_NE_H11 | Monthly EA updates of HADUK data |
| Ure | TPD_NE_H08 | Monthly EA updates of HADUK data |
| SwaleNE | TPD_NE_H07 | Monthly EA updates of HADUK data |
| Rye | TPD_NE_H13 | Monthly EA updates of HADUK data |
| Ouse | TPD_NE_H10 | Monthly EA updates of HADUK data |
| Nidd | TPD_NE_H09 | Monthly EA updates of HADUK data |
| Don | TPD_NE_H17 | Monthly EA updates of HADUK data |
| DerwentNE | TPD_NE_H14 | Monthly EA updates of HADUK data |
| Dales NSTribs | TPD_NE_H12 | Monthly EA updates of HADUK data |
| Hull and Humber | TPD_NE_H18 | Monthly EA updates of HADUK data |
| Calder | TPD_NE_H16 | Monthly EA updates of HADUK data |
| Aire | TPD_NE_H15 | Monthly EA updates of HADUK data |
| Yorkshire regional | Yorkshire regional | EA data request-HADUK |
| Nidd group | North West catchment | EA data request-HADUK |
| Washburn group | North catchment | EA data request-HADUK |
| Walshaw group | South West catchment |  |
| Leeshaw/Leeming | North West catchment | EA data request-HADUK |
| Ponden/Lower Laithe | North West catchment | EA data request-HADUK |
| Grimwith | North West catchment | EA data request-HADUK |
| Brownhill_Digley_group | South West catchment | EA data request-HADUK |
| Don_Valley_group | South catchment | EA data request-HADUK |
| Little_Don_Valley_group | South catchment | EA data request-HADUK |
| Loxley_group | South catchment | EA data request-HADUK |
| Rivelin_Redmires | South catchment | EA data request-HADUK |
| Ryburn_us_slitheroe_bridge | South West catchment | EA data request-HADUK |

Table 1: Rainfall Catchments

Figure 1: River and reservoir catchments

## Rainfall and climate data analyses

The following analyses are included in our rainfall and climate analyses:

- Standardised Precipitation Index (SPI) calculations

- Rainfall ranking

- Rainfall probability bands

- Rainfall percentage of Long Term Average

- Reservoir Stocks

- Soil Moisture Deficit maps over time

- SPI maps from CEH Water Resources Portal

- River flow maps and graphs from CEH Water Resources Portal

## Rainfall as percentage of Long Term Average (LTA)

Figure 2 shows the monthly rainfall values as percentage of the 1961-1990 long term average (LTA) for regional Yorkshire rainfall series from October 2021.  In this graph, red bars show rainfall below the LTA, and blue bars show monthly rainfall above the LTA.

It is clear that there has been a substantial rainfall deficit since rainfall has been below LTA in all but 3 months since October 2021, with September 2022 being only just above LTA for the regional series.  A dry autumn 2021 meant that reservoirs were below the normal control line until the very wet February, but rainfall has been below average since then.  In both June and July, much of the rainfall occurred in the last week and 10 days of the month respectively, with dry conditions followed by a short period of rainfall, much of which soaked into dry ground.

Figure 2: 2021-22 Regional rainfall as percentage of long term average (1961-1990 LTA)

Graphs for all of the river and reservoir catchments listed in Table 1 are shown in Appendix 1.  These all show a dry November-January period, wet February, and dry period from March onwards.  The graphs in Appendix 1 show only October 2021 and February 2022 have had significantly higher than average rainfall since October 2021, and September 2022 rainfall has been close to average.  It is worth noting that our reservoirs are located in very wet areas, and although the actual rainfall has been similar in some of the larger catchments and some of the small reservoir catchments, the proportion of what we would expect in those areas is low in comparison to what the systems are used to.  The regional data is HADUK data supplied by the Environment Agency for the Yorkshire region.  We have shown this for illustrative purposes in the main part of this document, and show data for the selected river and reservoir catchments in Appendix 1.  September has been just above average regionally, although for the river and reservoir catchments shown in Appendix 1, rainfall is close to the LTA in September – just below in some areas and just above in others.

Appendix 1 also contains a table showing the March-August and March-September rainfall as percentage of the LTA for each catchment.

## Ranking of rainfall

Table 2 shows the rank of rainfall for the Yorkshire region in the 131 or 132 year period of record, for periods of up to 12 months ending from October 2021 to September 2022.  (131 years for data to 2021, 132 years for data to 2022).  Each cell is shaded to indicate how the 2022 rainfall is classified according to the Cunnane plotting position (see Appendix 2).  Tables for the other rainfall catchments listed in Table 1 are shown in Appendix 2a.

Table 2 shows that the 6 and 7 months to September are ranked 5th and 6th in the Yorkshire region since 1891, and the 5 and 6 months until August are the 4th and 3rd ranked respectively.  These, and the other periods shaded red, are classified as “exceptionally low” rainfall according to the Cunnane plotting position.

The 6 months ending in August had exceptionally low rainfall for all catchments. They were the driest in the period of record for the Walshaw Dean, Brownhill Digley and Little Don Valley catchments, and 2nd driest for Calder river catchments and the Don Valley Group reservoir catchments.  This illustrates just how extreme the weather of 2022 has been in some of our Pennine catchments where our supply reservoirs are located.  Using the Cunnane plotting position, the driest in the 132 year record has a return period of 220 years, and the second driest has a return period of 82 years, although to more accurately calculate the return period an extreme value analysis (EVA) distribution would need to be fitted.

September rainfall has been close to average, at 105% regionally.  This close to average rainfall has helped to replenish rainfall deficits, and the 6 and 7 month periods ending in September are still ranked in the top 6 and so in the exceptionally low category in several of the reservoir catchments and some of the river catchments, and is still in the notably low category for the remaining catchments.

|  | Rank of 2022 rainfall in period since 1891 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | duration |  |  |  |  |  |  |  |  |  |  |  |
| end month | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 10 | 94 | 74 | 51 | 74 | 47 | 87 | 60 | 60 | 72 | 97 | 106 | 95 |
| 11 | 27 | 51 | 51 | 36 | 49 | 33 | 69 | 42 | 44 | 54 | 84 | 94 |
| 12 | 57 | 32 | 54 | 44 | 32 | 47 | 32 | 64 | 41 | 40 | 49 | 78 |
| 1 | 9 | 22 | 13 | 25 | 21 | 14 | 24 | 13 | 44 | 25 | 27 | 37 |
| 2 | 127 | 90 | 80 | 63 | 71 | 61 | 49 | 62 | 43 | 75 | 56 | 54 |
| 3 | 41 | 116 | 72 | 68 | 49 | 59 | 58 | 45 | 57 | 37 | 70 | 49 |
| 4 | 27 | 20 | 91 | 52 | 56 | 36 | 45 | 45 | 33 | 45 | 33 | 55 |
| 5 | 64 | 25 | 16 | 87 | 47 | 48 | 33 | 39 | 44 | 34 | 43 | 27 |
| 6 | 35 | 29 | 14 | 12 | 66 | 30 | 36 | 25 | 33 | 33 | 20 | 35 |
| 7 | 26 | 14 | 14 | 5 | 8 | 41 | 17 | 19 | 12 | 22 | 19 | 17 |
| 8 | 10 | 9 | 5 | 6 | 4 | 3 | 19 | 6 | 11 | 6 | 9 | 9 |
| 9 | 84 | 29 | 18 | 10 | 9 | 5 | 6 | 23 | 12 | 15 | 10 | 12 |

Key

| Exceptionally low | Notably low | Below normal | Normal | Above normal | Notably high | Exceptionally high |
| --- | --- | --- | --- | --- | --- | --- |

Table 2: Yorkshire Regional rainfall ranking of 2022 rainfall (from 1891-2022)

Appendix 2a shows that 2022 ranks in the top 10 of the period of record for some of the larger river catchments, but for some of the small reservoir catchments in the south and west of the region, it is higher ranked.

Appendix 2b shows tables of the driest ranked event for each of the rainfall catchments, and Appendix 2c shows the last year that was drier than 2022 for each month ending and duration.

Appendix 2b shows that for many ending months and durations, 1995 was the driest, but for a few locations and durations 2022 is now the driest.

Appendix 2c shows that for many durations ending in August and September, 2022 is the driest since 1995/1996 for the regional rainfall series, showing how dry this year has been in comparison to our previous most severe drought.

## Soil Moisture Deficit (SMD)

Rainfall is just one of the many indicators of drought, although it is the primary one.  We have used the Met Office Rainfall and Evaporation Calculation System (MORECS) to show Soil Moisture Deficit (SMD) throughout the region. Ground conditions were fully saturated in all of the region in February after high rainfall, but low rainfall and high temperatures led to a relatively early increase in SMD, with values showing very dry conditions of over 100mm (shaded dark orange in the maps below) for one area at the start of June and over 110mm in almost all areas by the start of July.  It has been extremely dry earlier in the year compared to other years. The maps below show the SMD from May 2022.

Figure 3: Soil Moisture Deficit for Yorkshire 2022

## Standardised Precipitation Index (SPI)

The Standardised Precipitation Index (SPI) values can be interpreted as the number of standard deviations by which the observed anomaly deviates from the long-term mean. The SPI can be calculated for differing periods using monthly input data.  We have calculated the SPI for all the monthly rainfall data listed in Table 1 for a 1 month, 3 month, 6 month and 7 month duration, and these are shown in Appendix 3.  We have followed the methodology described in report Environment Agency Standardised Precipitation Index (SPI) Tool.  We have not used the Environment Agency Tool, as the spreadsheet macros do not work on our computers, but we have used the methodology described and our results are the same as those provided by the Agency.

Table 3 shows the SPI values calculated for September for our rainfall catchments for the 1, 3 and 6 month durations, and for the March-September 7 month period.  The close to average rainfall in September has reduced the severity of the dryness of the SPIs.  For the 7 months until the end of September, most catchments are classified as either severely or extremely dry.

| Catchment | 1 month September SPI | 3 month July-September SPI | 6 month April-September SPI | 7 month March-September SPI |
| --- | --- | --- | --- | --- |
| Wharfe | 0.31 | -1.11 | -1.49 | -1.79 |
| Ure | 0.43 | -0.76 | -1.25 | -1.54 |
| SwaleNE | 0.87 | -0.52 | -1.12 | -1.26 |
| Rye | 0.66 | -0.95 | -1.52 | -1.62 |
| Ouse | 0.50 | -1.34 | -1.66 | -1.62 |
| Nidd | 0.46 | -0.97 | -1.35 | -1.51 |
| Don | 0.30 | -0.99 | -1.57 | -1.62 |
| DerwentNE | 0.31 | -1.42 | -1.91 | -1.98 |
| Dales NSTribs | 0.62 | -0.93 | -1.58 | -1.56 |
| Hull and Humber | 0.36 | -1.36 | -1.89 | -2.01 |
| Calder | 0.09 | -1.33 | -1.88 | -2.23 |
| Aire | 0.17 | -1.37 | -1.81 | -2.09 |
| Yorkshire regional | 0.37 | -1.18 | -1.78 | -1.95 |
| Nidd group | 0.47 | -0.61 | -0.98 | -1.35 |
| Washburn group | 0.33 | -1.15 | -1.62 | -1.90 |
| Walshaw group | 0.03 | -1.50 | -2.07 | -2.57 |
| Leeshaw/Leeming | 0.07 | -1.37 | -1.80 | -2.22 |
| Ponden/Lower Laithe | 0.04 | -1.46 | -1.92 | -2.37 |
| Grimwith | 0.31 | -0.83 | -1.14 | -1.49 |
| Brownhill_Digley_group | 0.07 | -1.22 | -1.79 | -2.21 |
| Don_Valley_group | 0.12 | -1.17 | -1.80 | -2.04 |
| Little_Don_Valley_group | 0.15 | -1.19 | -1.94 | -2.30 |
| Loxley_group | 0.28 | -0.95 | -1.67 | -1.91 |
| Rivelin_Redmires | 0.24 | -0.83 | -1.47 | -1.64 |
| Ryburn_us_slitheroe_bridge | -0.01 | -1.16 | -1.74 | -2.17 |

Key

| Extremely dry | Severely dry | Moderately dry | Near normal | Moderately wet | Severely wet | Extremely wet |
| --- | --- | --- | --- | --- | --- | --- |

Table 3: September SPI values for Yorkshire catchments

Appendix 3 shows maps of the SPI from the Centre for Ecology and Hydrology Water Resources Portal for August and September for the 1, 3 and 6 month durations.  The catchment for the River Wharfe is outlined.  The CEH SPI calculates SPI in the same way as the EA recommended method, but currently has a slightly different rainfall dataset, and uses a different method to fit the Gamma distribution, so the calculated values may differ slightly to those shown below, but the maps provide a good visual indication of the SPI.

## River Flows

The graphs below taken from our weekly Water situation report show river flows in 2022 compared to recent years and 1995 and 1996- our worst drought years.  These show that river flows have been low in 2022, and for some rivers they have been at low levels relatively early in the year, even when compared to some of our worst drought years such as 1995.

| Figure 4:River Derwent flows Flows in the River Derwent during June and July have been as low as they were in 1995 and 1996. |  |  |
| --- | --- | --- |

Figure 5:River Hull flows

Flows in the River Hull have had very low periods in July and August 2022.

Figure 6:River Ouse flows

Flows in the River Ouse were lower in periods of June and July 2022 than they were in 1995 and 1996.

Figure 7:River Ure flows

Flows in the River Ure were lower in periods of June and July 2022 than they were in 1995 and 1996.

Figure 8:River Wharfe flows

The graphs below taken from the CEH Water Resources portal show river flows and the accompanying rainfall for the River Ouse.  They show that for both June and July 2022, although the monthly rainfall is only a little below average, the river flows were very low for periods of time and then recovered when rainfall has occurred late in the month. Intermittent rainfall and high SMD have meant that more of the rainfall than usual is soaked up by the dry ground.  It can be seen that at times in July, river flows were “exceptionally low” (see Appendix 4 for a description of the regime bandings shown below), but the rainfall at the end of the month brought flows into the “above normal” range, meaning that average monthly flows were in the normal range. River flows in August and September have responded to intermittent rainfall, and been in bands normal to exceptionally low.

Figure 9: CEH River flows and banding: River Wharfe

Figure 10: CEH Rainfall station catchment rainfall:River Wharfe

## Reservoir stocks

The graphs below show the stocks of our regional reservoir group and our 5 major reservoir groups, East, North, North West, South and South West for the last 12 years and selected other dry years – 2003, 1995 and 1996.  They all show 2022 stocks falling earlier and more steeply than in most recent years. In 2020 stocks fell earlier, but then recovered. Since mid June 2022 stocks have been lower than at the same time of year in previous years, other than the 1995 drought.  This further illustrates how dry 2022 has been in comparison to other years.  Area reservoir stocks and group and individual reservoir stocks are shown in the supporting information documents for each set of drought permit applications.

Figure 11: Regional reservoir stocks

Figure 12: North West group reservoir stocks

Figure 13: South West group reservoir stocks

Figure 14: North group reservoir stocks

Figure 15: South group reservoir stocks

Figure 16: East reservoir stocks

## Weather Forecast

The Met Office long range weather forecast (18 October) is for unsettled weather with rain in the southwest with a shift towards drier and more settled weather by mid November.

There is no clear signal for an end to the dry weather we have been experiencing, and so we are making preparations on the assumption that the dry weather will continue.  If we have a repeat of 1995 autumn and winter rainfall, rainfall totals from the start of March 2022 would be among the lowest in our 132 year record.

## Conclusions

The spring and summer of 2022 has been exceptionally hot and dry.

For several of our catchments, the 6 month period to August was among the driest since records began in 1891, although the July figures disguise just how dry most of July was, with the majority of the rainfall in July occurring towards the end of the month.  The SPI and ranking analyses indicate conditions were extremely or exceptionally dry in several catchments.

The low rainfall in the summer was accompanied by periods of extremely hot weather, and this combination of hot dry weather resulted in very high SMD values throughout the region, as well as extremely high demands.

River flows have been low due to the low rainfall, and high SMD, meaning that when there has been intermittent rainfall, it has not resulted in significant runoff to rivers.

The early autumn has seen rainfall return to normal , with close to average rainfall in September, but the substantial rainfall deficit of the spring and summer months has a lasting impact on our reservoir stocks, although river flows are now recovering.

It is the combination of low rainfall and high temperatures (so high evaporation) that has reduced the inflows into our rivers and reservoirs.  Our Pennine reservoirs are situated in wet uplands, and when the inflows are below normal for long periods of time, our crucial resources are at risk even when the rainfall totals we have seen have been far higher than the south of England.  The record breaking high temperatures have also led to very high demands, which accelerated the rate of reservoir drawdown.

Our analyses of rainfall until the end of August demonstrate that the current deficiency in supplies is a direct result of the exceptional shortage of rainfall.  Updating these analyses to include September data has reduced the severity of the rainfall deficit, but still demonstrates that we have experienced an exceptional shortage of rainfall that has led to the deficiency in supplies.  September and early October rainfall has improved the situation, but the effects of the exceptionally dry weather still persist, resulting in the need for drought measures.

Our analyses show that if we had a repeat of inflows from most years from the current date (October 2022), reservoir stocks would recover, but if we had a repeat of some of the drier years in our historic record, reservoir stocks would fall to very low levels, and fail to refill in winter.   Our analyses also show that the temporal pattern of rainfall we have had in 2022 has been similar to that in 1995 (We have had 66% of LTA March-July in 2022 compared to 61% for the same period in 1995).   We therefore think it prudent to plan for an autumn and winter like the one that followed the hot dry summer of 1995, as both have been a result of similar prevailing meteorological conditions.

## References

CEH Water Resources Portal- https://eip.ceh.ac.uk/hydrology/water-resources/

Environment Agency, Environment Agency Standardised Precipitation Index (SPI) Tool, version 2.0.

Met Office/CEH and Environment Agency– HADUK Datasets

MET OFFICE 2022, https://www.metoffice.gov.uk/about-us/press-office/news/weather-and-climate/2022/driest-july-in-england-since-1935

Met Office Rainfall and Evaporation Calculation System. This is a generic name for Met Office services involving the routine calculation of soil moisture and evaporation for Great Britain and uses a grid 40 x 40km squares.

# Appendices

## Appendix 1: Rainfall in comparison to LTA

|  | Cumulative rainfall as % of LTA |  |
| --- | --- | --- |
| Catchment | March-August | March-September |
| Wharfe | 63% | 69% |
| Ure | 64% | 71% |
| Swale | 65% | 77% |
| Rye | 61% | 71% |
| Ouse | 62% | 70% |
| Nidd | 64% | 71% |
| Don | 61% | 66% |
| DerwentNE | 59% | 65% |
| Dales NSTribs | 62% | 72% |
| Hull and Humber | 55% | 63% |
| Calder | 56% | 61% |
| Aire | 58% | 63% |
| Regional | 60% | 67% |
| Nidd reservoir group | 65% | 71% |
| Washburn group | 58% | 65% |
| Walshaw group | 52% | 57% |
| Leeming/Leeshaw | 55% | 60% |
| Grimwith | 63% | 69% |
| Ponden/Lower Laithe | 54% | 59% |
| Brownhill_Digley_group | 55% | 60% |
| Don_Valley_group | 56% | 61% |
| Little_Don_Valley_group | 52% | 58% |
| Loxley_group | 58% | 63% |
| Rivelin_Redmires | 62% | 67% |
| Ryburn_us_slitheroe_bridge | 58% | 61% |

## Appendix 2a: Ranking of Rainfall Results

The following tables show the rank of the rainfall from October 2021 until September 2022 for the 131 (October-December 2021) or 132 (2022) years of data, for periods of 1 to 12 months duration.

In the tables below values have been shaded according to their category as calculated using Cunnane’s plotting position, which calculates the probability of one of the ranked values being lower than expected.

The equation below shows the formula for the Cunnane plotting position, and the table shows the thresholds for the different categories.
Cunnane plotting position= Rank-0.4n+0.2

| Category | Probability of value being surpassed by lower value P(X) | Probability of occurrence |
| --- | --- | --- |
| Exceptionally high | >0.95 | 0.05 (5%) |
| Notably high | 0.87 – 0.95 | 0.08 (8%) |
| Above normal | 0.721 – 0.869 | 0.15 (15%) |
| Normal | 0.28 – 0.72 | 0.44 (44%) |
| Below normal | 0.131 – 0.279 | 0.15 (15%) |
| Notably low | 0.05 – 0.13 | 0.08 (8%) |
| Exceptionally low | <0.05 | 0.05 (5%) |

## Appendix 2b: Driest period on record

## Appendix 2c: Last Year drier than 2021/22 for durations 1-12 months

The table below shows the last year drier than 2021/22. Cells are highlighted if the last year drier than this year was before 1996.  Cells are blank and highlighted if this year is the driest on record.

## Appendix 3: SPI graphs and maps from CEH Water Resources portal

## Appendix 4: River flow graphs from CEH Water Resources portal

The river flow graphs in this appendix are taken from the CEH Water resources portal, and are shown in river flow bandings as used by the Environment Agency.  The associated rainfall graphs are only plotted until the end of July, as are not available on the CEH portal currently.

The Water Resources daily river flow banding procedure follows that developed by the Environment Agency, in order to provide information and visualisations consistent with their Water Situation Reports (https://www.gov.uk/government/collections/water-situation-reports-for-england). Essentially, values for 7 flow bands are calculated for each day (“Exceptionally high”, “Notably high”, “Above normal”, “Normal” through to “Exceptionally low”) corresponding to thresholds of 5, 13, 28, 72, 87, and 95 percentiles (defining bands for 0-5, 5-13, 13-28. etc.). Values for a given day are calculated using all flows within a 30-day window around that day (15 days before and 14 days after). The values are pooled and the percentile values calculated for each day. All data is used from the beginning of the record up to a standard end date, which is updated on a regular basis (currently 31/12/2017). Only series with 20 years of data (up to this end date) are included. Note that the validated gauged daily flow data from the NRFA is used to provide the bands, whereas the time series data plotted against the bands is live data from EA / SEPA APIs, which may differ slightly from that in NRFA archives.